- PART 1 - GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following:

          - Carpet.

          - Carpet cushion.

          - Carpet transitions.

      - SUBMITTALS

        - Product Data:  For the following, including installation recommendations for each type of substrate:

          - Carpet:  For each type indicated.  Include manufacturer's written data on physical characteristics, durability, and fade resistance.

          - Carpet Cushion:  For each type indicated.  Include manufacturer's written data on physical characteristics and durability.

          - Warranty:  Copy of warranty for each type indicated.

        - Shop Drawings:  Show the following:

          - Columns, doorways, enclosing walls or partitions, built-in cabinets, and locations where cutouts are required in carpet.

          - Carpet type, color, and dye lot.

          - Locations where dye lot changes occur.

          - Seam locations, types, and methods.

          - Type of subfloor.

          - Type of installation.

          - Pattern type, repeat size, location, direction, and starting point.

          - Pile direction.

          - Type, color, and location of edge, transition, and other accessory strips.

          - Transition details to other flooring materials.

          - Type of carpet cushion.

        - Samples:  For each of the following products and for each color and texture required.  Label each Sample with manufacturer's name, material description, color, pattern, and designation indicated on Drawings and in schedules.

          - Carpet:  24-inch- (600-mm-) square Sample.

          - Exposed Edge, Transition, and other Accessory Stripping:  12-inch- (300-mm-) long Samples.

          - Carpet Cushion:  6-inch- (150-mm-) square Sample.

      - DELIVERY, STORAGE, AND HANDLING

        - Comply with CRI 104, Section 5, "Storage and Handling."

      - PROJECT CONDITIONS

        - Comply with CRI 104, Section 7.2, "Site Conditions; Temperature and Humidity" and Section 7.12, "Ventilation."

        - Environmental Limitations:  Do not install carpet and carpet cushion until wet work in spaces is complete and dry, and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use.

        - Where demountable partitions or other items are indicated for installation on top of carpet, install carpet before installing these items.

      - WARRANTY

        - Special Warranty for Carpet:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of carpet installation that fail in materials or workmanship within specified warranty period.

          - Warranty does not include deterioration or failure of carpet due to unusual traffic, failure of substrate, vandalism, or abuse.

          - Failures include, but are not limited to, more than 10 percent loss of face fiber, edge raveling, snags, runs, loss of tuft bind strength, excess static discharge, and delamination.

          - Warranty Period:  10 years from date of Substantial Completion.

        - Special Warranty for Carpet Cushion:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of carpet cushion installation that fail in materials or workmanship within specified warranty period.

          - Warranty includes consequent removal and replacement of carpet and accessories.

          - Warranty does not include deterioration or failure of carpet cushion due to unusual traffic, failure of substrate, vandalism, or abuse.

          - Failure includes, but is not limited to, permanent indentation or compression.

          - Warranty Period:  10 years from date of Substantial Completion.

C. 	Special Warranty for Installation Materials:  Provide single source installation system including moisture mitigation, surface prep and patches, and carpet adhesives with a manufacturer’s system warranty for a period of no less than 10 years.

1. 	Warranty does not include deterioration or failure of carpet due to unusual traffic, failure of substrate, vandalism, or abuse.

2.  	Warranty covers damage from moisture vapor (when moisture vapor membrane is used as part of installation system), bond and performance of patch or self leveling cement, primer and carpet adhesive including labor and materials for a period of no less than 10 years as spelled  out in manufacturers installation system warranty.

- PRODUCTS

      - CARPET

        - Products:  Subject to compliance with requirements, provide the following:

          - As selected by Architect.

          - Color as selected by Architect.

          - Pattern as selected by Architect

          - Acceptable Manufacturers

      - CARPET CUSHION

        - Products:  Subject to compliance with requirements, provide the following:

          - As selected by Architect

            - Thickness:  3/8” maximum

            - Density:  6 pound minimum

            - FHA UM-72

      - INSTALLATION ACCESSORIES

        - Trowelable Leveling and Patching Compounds:  Latex-modified, hydraulic-cement-based formulation provided as a single source installation system (See section 3540)

          - Acceptable Manufacturers.

            - TEC®/H.B. Fuller Construction Products Inc.:  1105 S. Frontenac Street, Aurora, IL 60504, web: www.tecspecialty.com.

        - Adhesive:  Water-resistant, mildew-resistant, nonstaining, 0 VOC, withstands up to 8 lb. MVER or 86% RH type to suit products and subfloor conditions indicated, that complies with flammability requirements for installed carpet and is  or provided by installation system manufacturer.

          - TEC® Premium Fast Grab Carpet Adhesive (TA-704). Install as recommended by TEC®/H.B. Fuller Construction Products Inc.  www.tecspecialty.com.

          - TEC® Double Stick Carpet Adhesive (TA-702) for pattern matched carpets using direct bond or double stick installation method. Install as recommended by TEC®/H.B. Fuller Construction Products Inc.  www.tecspecialty.com.

          - TEC® The Perfect Putdown® Premium Carpet and Flooring Adhesive (TA-717) for mineral felt backed, fiber backed, and paperbacked carpet. Install as recommended by TEC®/H.B. Fuller Construction Products Inc.  www.tecspecialty.com.

        - Tackless Carpet Stripping:  Water-resistant plywood, in strips as required to match cushion thickness and that comply with CRI 104, Section 12.2.

        - Seam Adhesive:  Hot-melt adhesive tape or similar product recommended by carpet manufacturer for sealing and taping seams and butting cut edges at backing to form secure seams and to prevent pile loss at seams.

        - Metal Edge Strips:  Extruded aluminum with mill finish of width shown, of height required to protect exposed edge of carpet, and of maximum lengths to minimize running joints.

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for maximum moisture content, alkalinity range, installation tolerances, and other conditions affecting carpet performance.  Examine carpet for type, color, pattern, and potential defects.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - General:  Comply with CRI 104, Section 7.3, "Site Conditions; Floor Preparation," and with carpet manufacturer's and installation system manufacturer’s written installation instructions for preparing substrates.

        - Use trowelable leveling and patching compounds, according to installation system manufacturer's written instructions, to fill cracks, holes, depressions, and protrusions in substrates.  Fill or level cracks, holes and depressions 1/8 inch (3 mm) wide or wider, and protrusions more than 1/32 inch (0.8 mm), unless more stringent requirements are required by installation system manufacturer's written instructions.

        - Broom and vacuum clean substrates to be covered immediately before installing carpet.

      - INSTALLATION

        - Comply with CRI 104 and carpet, carpet cushion, and installation system manufacturers' written installation instructions for the following:

          - Stretch-in Installation:  Comply with CRI 104, Section 12, "Stretch-in Installation."

            - Install in________.

          - Direct-Glue-Down Installation: Comply with CRI 104, Section 9, “Direct Glue-Down Installation.”

            - Install in _______.

        - Comply with carpet manufacturer's written recommendations and Drawings for seam locations and direction of carpet; maintain uniformity of carpet direction and lay of pile.  At doorways, center seams under the door in closed position.

        - Do not bridge building expansion joints with carpet.

        - Cut and fit carpet to butt tightly to vertical surfaces, permanent fixtures, and built-in furniture including cabinets, pipes, outlets, edgings, thresholds, and nosings.  Bind or seal cut edges as recommended by carpet manufacturer.

        - Extend carpet into toe spaces, door reveals, closets, open-bottomed obstructions, removable flanges, alcoves, and similar openings.

        - Maintain reference markers, holes, and openings that are in place or marked for future cutting by repeating on finish flooring as marked on subfloor.  Use nonpermanent, nonstaining marking device.

        - Install pattern parallel to walls and borders to comply with CRI 104, Section 15, "Patterned Carpet Installations" and with carpet manufacturer's written recommendations.

        - Comply with carpet cushion manufacturer's written recommendations.  Install carpet cushion seams at 90-degree angle with carpet seams.

      - CLEANING AND PROTECTING

        - Perform the following operations immediately after installing carpet:

          - Remove excess adhesive, seam sealer, and other surface blemishes using cleaner recommended by carpet manufacturer.

          - Remove yarns that protrude from carpet surface.

          - Vacuum carpet using commercial machine with face-beater element.

        - Protect installed carpet to comply with CRI 104, Section 16, "Protection of Indoor Installations."

        - Protect carpet against damage from construction operations and placement of equipment and fixtures during the remainder of construction period.  Use protection methods indicated or recommended in writing by carpet manufacturer and carpet cushion manufacturer.

END OF SECTION

Printed in U.S.A.

©2011 H.B. Fuller Construction Products Inc.

Form #  SPEC09680 N0411-1